# e-CALLISTO Manufacturing Check List

| Date | Author | Reference | Version | Release |
| --- | --- | --- | --- | --- |
| 2008-04-02 | Chr. Monstein | MAIT.doc | 1.0 | Draft |
| 2009-09-07 | Chr. Monstein | MAIT.doc | 1.4 | AGV test |
| 2012-12-18 | Chr. Monstein | CheckList.doc | 1.5 |  |
| 2014-04-22 | Chr. Monstein | CheckList.doc | 1.6 | Change of seq. |
| 2018-03-15 | Chr. Monstein | Minor changes | 1.7 |  |
| 2024-03-02 | Chr. Monstein | Minor changes | 1.9 | +LED |

Spectrometer identification: eC…..

| Test # | Work description | Remarks/ value | Date Signature |
| --- | --- | --- | --- |
| 0 | Supply the board with 12V and current limited to 0,1A max, measure actual current Board-status only passive components, transistor, transformers but no IC’s | …....….mA |  |
| 1 | Supply the board with 12V and current limited to 0,5A max, measure actual current Fully mounted board | ……….mA |  |
| 2a | Preliminary functional test in test-frame with callisto.exe (internal clock) and simple wipe antenna, check spectrum level which should be ~50% FS |  |  |
| 2b | Check all FPU digits using FPU-Test box (LEDs) and appropriate frequency program or command sent by HyperTerminal or using /FocuscodeTest/FC_Test.exe |  |  |
| 3 | Measure the low pass filter characteristics of the AGC-filter using AGC_PWM/iecbus.exe No need to save png | Minimum at …. digit |  |
| 6 | Adjust IF2- and IF1-band pass filter to maximum using digitizer-mode at 408.0 MHz Set Marconi rf-generator to 408 MHz and power = -70 dBm ….. -90 dBm |  |  |
| 7 | Measure the overall bandwidth using program ieee488_Bandwidth_plott/iecbus.exe -> save plot as png and check it |  |  |
| 8 | Measure lowest discernible sensitivity using ieee488_Gain_Sensitivity_plott/iecbus.exe -> save plot as png and check it. Power level: [-118 …(1…2) …. -104] dBm |  |  |
| 9 | Measure SFDR using ieee488_SFDR_plott/iecbus.exe -> save plot as png and check |  |  |
| 10 | Measure dynamic range using ieee488_DynamicRange_plott/iecbus.exe -> save plot as png and check |  |  |
| 11 | Measure noise figure at all frequencies using NoiseFigure/NF.exe -> save plot as png Average value should be 9+/-1 dB |  |  |
| 12 | Check external clock 1MHz TTL5V from external function generator or DCF77 clock Check signal with scope before connecting to Callisto! |  |  |
| 13 | Check detector analogue output using Digital-Mulitmeter or Scope, expect 0.9…1.3V | ……...Volt |  |
| 14 | Check brightness of front-LED. Depeding on color, R needs to be adjusted |  |  |
| 15 | Terminate receiver with 50Ω and measure > 1 hour. Take out quiet channel near 608 MHz using “Fits2Ascii” and feed light curve into “AlaVar” to determine Allan-time. Write Allan-time into the field remark right. -> save plot / report | ……… sec |  |
| 16 | Do the endurance test for 1 week to ensure long term behaviour |  |  |
| 17 | Copy *.png to web-folder: https://e-callisto.org/Qualification/applidocs.htm and update website using Visual Studio Code |  |  |